NCP1568 Simetrix model

- The schematic and simulation is ready to use with SIMetrix 8.1.

- The user should be able to simulate by pressing the run button.

- If the NCP1568 is accidentally deleted the user must take the following steps or down load the schematic again

  - Install the Library

    - File > Model Library > add/ remove Libraries

    - Point the location that the NCP1568 Simulation files were saved during download.

    - Ensure models are added to the current selected libraries by using the arrows on the select libraries tab.

    - To place the NCP1568 go to file > Model Library > Symbols manager> NCP1568.sxslb under symbol click Vendor>PSU Controller> NCP1568> Place part

- If the model is not running you may need to Include the ASCII model library in the simulation. One easy way to do this is with a .include statement.

  - Copy the following and paste into the F11 window after the .SIMULATOR DEFAULT statement

    - .include NCP1568_encrypted.lb

Exclusions from the model

- The VCC power is not modeled

- The HV startup is not molded

- The FLT function is not molded

| Modifier | Modification Date | Modification |
| --- | --- | --- |
| Ajay Hari | 11-02-2018 | Initial Release |